| 17.033 |  | Rubber Seat |  |
| --- | --- | --- | --- |
| Size, mm: | 240x440x1482 | Weight, kg: | 5.855 |
| Material characteristics |  |  |  |
| Materials: metal, rubber. Complete set: rubber suspension – 1 pc., fastener kit – 1 pc. The design consists of: Anti-vandal seat for children's swings. Made of thermoplastic elastomer, reinforced with an aluminum plate. Suspension made of galvanized round-link chain, 6 mm link. Fastened to the seat through galvanized bushings with M8 x 20 thread. Fixed at the bottom with a washer and a self-locking nut M8. Protruding parts of threaded connections and open parts of pipes are covered with plastic plugs Coating of elements: metal - powder zinc primer, powder paint. |  |  |  |