| 25.019 |  | Trampoline square 1780 х 1780 (rubber) |  |
| --- | --- | --- | --- |
| Size, mm: | 1780x1780x380 | Weight, kg: | 210 |
| Material characteristics |  |  |  |
| Materials: metal, rubber. Package includes: trampoline - 1 pc. The built-in square trampoline is supplied as a single element, ready for installation, and includes a welded frame made of galvanized steel sheets 3-4 mm thick, a reinforced rubber sheet with a tread 7-8 mm thick and galvanized springs 140 mm long with a load of up to 55 kg. The springs are attached through anti-friction bronze bushings, eliminating wear and creaking, which are subject to replacement every 3 years. - metal - hot zinc |  |  |  |