| MG3510 |  | Sports equipment Jumping ladder |  |
| --- | --- | --- | --- |
| Size, mm: | 2490x3079x1055 | Weight, kg: | 0 |
| Material characteristics |  |  |  |
| Materials: softwood - larch, metal, reinforced polypropylene rope, stainless steel. Package includes: support post - 6 pcs., crossbars - 3 pcs., rope ladder - 1 pc., fastener kit - 1 pc. The support posts are made of rounded larch timber with a diameter of 140 mm. The crossbars are also made of rounded larch timber with a diameter of 90 mm. The sports equipment includes an inclined rope net made of six-strand reinforced polypropylene rope d = 16 mm. The complex uses stainless steel fasteners. Protruding parts of threaded connections and open parts of pipes are covered with plastic plugs. Coating of elements: - metal - zinc powder primer, powder paint; - timber - glazing primer, water-dispersion varnish. |  |  |  |