| MG4524 |  | Sports equipment for workout |  |
| --- | --- | --- | --- |
| Size, mm: | 2195x3760x2800 | Weight, kg: | 202.1 |
| Material characteristics |  |  |  |
| Main materials: softwood lumber, metal, stainless steel, stainless steel fasteners. Package includes: 6 embedded parts; 6 support posts; 1 parallel bar crossbar; 2 horizontal bar crossbars; 1 set of fasteners. The support posts are made of rounded larch lumber with a diameter of 140 mm. The profile is round, with a corner radius of R10. The horizontal bar crossbar between the posts is made of stainless steel pipe with a diameter of 33.7 mm and a wall thickness of 2 mm. The parallel bar crossbars between the posts are made of stainless steel pipe with a diameter of 42.4 mm and a wall thickness of 3 mm. The set uses stainless steel fasteners. Protruding threaded connections and mounting holes are covered with plastic plugs. Finish: - Metal - zinc powder primer, powder paint; - timber - glazing primer, water-dispersion varnish. |  |  |  |