| MG4531 |  | Sports equipment for workout |  |
| --- | --- | --- | --- |
| Size, mm: | 1720x7450x2800 | Weight, kg: | 434.7 |
| Material characteristics |  |  |  |
| Main materials: softwood lumber, metal, stainless steel, stainless steel fasteners. Package includes: 10 embedded parts; 10 support posts; 2 monkey bars; 6 crossbars; 1 set of fasteners. The support posts are made of insulated, 140 mm diameter larch lumber. The profile is round, with a corner radius of R10. The monkey bars and crossbars between the posts are made of stainless steel pipe with a diameter of 33.7 mm and a wall thickness of 2 mm. The set uses stainless steel fasteners. Protruding threaded connections and mounting holes are covered with plastic plugs. Finish: - Metal: zinc powder primer, powder paint; - Lumber: translucent primer, water-based varnish. |  |  |  |