| 26.017 |  | Workout System Hand-walker |  |
| --- | --- | --- | --- |
| Size, mm: | 550x1810x2600 | Weight, kg: | 77 |
| Material characteristics |  |  |  |
| Materials: metal, powder paint, stainless steel, plastic. Contents: support posts – 2 pcs; snake handlebar – 1 piece; clamps – 1 set; fastening kit – 1 pc. The complex consists of 2 support pillars and a snake walker. The support pillars are made of metal pipe with a diameter of 108 mm and a wall thickness of 3.5 mm. The upper part of the pillars is closed with a plastic plug with a truncated cap. The snake handle is made of a stainless metal round pipe with a diameter of 32 mm and a wall thickness of 2 mm. The connection of the gymnastic element is made using collapsible clamps. There are rubber sealing inserts for reliable fixation - the clamp fits tightly to the pipe and does not slip during operation. The clamp consists of 2 elements that are fixed together with bolts and nuts. The maximum load on the crossbar is 500 kg. All existing metal parts, except stainless steel, are pre-cleaned, degreased and painted with polyester powder paint. All threaded fasteners are galvanized. |  |  |  |