| ML4525.1 |  | Workout MG-25.1 |  |
| --- | --- | --- | --- |
| Size, mm: | 740x5800x2630 | Weight, kg: | 234.3875 |
| Material characteristics |  |  |  |
| Materials: Softwood timber (pine), metal. Components: Support posts - 9 pcs, bench - 1 pc, climbing ladder - 1 pc, bars - 4 pcs, fastener kit - 1 set. Specifications: Support Posts: Made from round steel tubing with an outer diameter of 89 mm and a wall thickness of 3 mm. Bars: Constructed from round steel tubing with an outer diameter of 40 mm and a wall thickness of 3 mm. Climbing Ladder: Fabricated from round tubing with an outer diameter of 33.5 mm and a wall thickness of 2.8 mm. Connection Hardware: Metal components are secured using die-cast aluminum clamps. These clamps are designed for connecting perpendicular tubes, specifically for fixing a horizontal bar to a vertical 89 mm diameter post. Press Bench: The equipment includes a press bench. The bench is made from laminated pine timber (profile B) with dimensions of 100x100 mm. Fasteners: Stainless steel fasteners are used throughout the assembly. Safety Finishes: Protruding threads and open tube ends are covered with protective plastic caps. Surface Coating: Metal: Zinc-rich primer with a powder coating finish. Timber: Tinted primer followed by a water-based dispersion varnish. |  |  |  |