| 17.215 |  | Three-section metal swing |  |
| --- | --- | --- | --- |
| Size, mm: | 1460x9470x2640 | Weight, kg: | 0 |
| Material characteristics |  |  |  |
| The frame is a structure consisting of 8 supports made of metal round pipes and 3 crossbars fixed between them. Round pipes with a diameter of 89mm and a wall thickness of 3mm. Fastening elements: M12x130 bolts, self-locking nuts, washers, fasteners for suspension. Metal painting is done with polymer paint . All open parts of pipes and bolted connections are closed with plastic plugs. Installation is carried out by deepening the lower part of the rack to a depth of 0.6 m, followed by concreting. Frame elements: Support post – 8 pcs. Crossbar – 3 pcs. Hanging fasteners – 8 sets. Fastening kit – 1 pc. Plug 89 mm - 8 pcs. |  |  |  |