| 34.151 |  | Park bench (round) |  |
| --- | --- | --- | --- |
| Size, mm: | 3000x3000x450 | Weight, kg: | 130 |
| Material characteristics |  |  |  |
| Materials: metal, wood, powder paint. The frame of the bench is made of profile pipe with high-tech processing, painted with glossy polyester powder paint. Seat frame - metal profile pipe with a cross-section of 25mm x 25mm, wall thickness 1.5 mm, legs - metal profile pipe with a cross-section of 40mm x 20mm, wall thickness 1.5mm and 20mm x 20mm, wall thickness 1.5mm. The seat consists of bars measuring 95mm x 37mm x 450mm, made from AB class softwood. The bars are treated with antiseptics and painted with tinting impregnations. |  |  |  |