| 40.020 |  | Meteor |  |
| --- | --- | --- | --- |
| Size, mm: | 204x2920x3600 | Weight, kg: | 86.757 |
| Material characteristics |  |  |  |
| The span is a structure of two sections, a gate and a mortgage. The sections of the structure are made of profile metal rectangular pipes welded together with a cross-section of 40 mm x 20 mm and a wall thickness of 2 mm and profile metal square section 20 mm x 20 mm and a wall thickness of 1.5 mm; connecting plates made of sheet steel 3 mm thick are welded to the ends of the rectangular profile. Along the entire length of the upper section of the span, strips of metal 3 mm thick are welded at different levels. A stop-limiter for the gate made of 3mm thick sheet metal is welded on the lower section. The gate is made of profile metal pipes welded together rectangular cross-section 40 mm x 20 mm and wall thickness 2 mm and profile metal square section 20 mm x 20 mm and wall thickness 1.5 mm. Loops for hanging on a pole are welded along the length of the gate; on the other side there is a cutout for a latch. The lower section is connected to a mortgage made of a profile metal pipe with a square section of 40 mm x 40 mm and a wall thickness of 2 mm, flanges at both ends, with a bolted connection Comes with a latch. All open parts of the pipes are closed with plastic plugs of the appropriate size. The spans are painted with polymer paint. |  |  |  |