| 13.010 |  | Play System Santorini |  |
| --- | --- | --- | --- |
| Size, mm: | 2143x2833x1895 | Weight, kg: | 247.01 |
| Material characteristics |  |  |  |
| Main materials: softwood timber, laminated plywood with anti-slip coating, HPL plastic, metal, monolithic polycarbonate, stainless fasteners. Contents: embedded parts – 1 set, support posts – 12 pcs., roof – 2 sets, floors – 2 sets, fencing and decorative panels – 1 set, drawing board – 1 pc., bridge-transition – 1 pc., bench – 1 pc., “Shapes” panel – 1 pc., abacus – 3 pcs., intercom – 1 pc., set of fasteners - 1 pc. Design and color palette of equipment according to the sketch. The playhouse consists of 2 houses. The support posts are made of laminated coniferous timber with a cross-section of 80x80 mm. The profile is square, the corner radius is R10. The transition between the houses is presented in the form of a metal bridge made of profile pipes 50x25 mm and 40x20 mm with a wall thickness of 2 mm and steps made of laminated plywood 18 mm thick with an anti-slip coating. The floor of the houses is made of 18 mm thick laminated plywood with an anti-slip coating. The roof of the house is gable, made of a combination of frames made of HPL plastic with monolithic polycarbonate, onto which color coating is applied using UV printing. The handles are made of metal pipe with a diameter of 26.8 mm and a wall thickness of 2.8 mm. Metal crossbars are made of a round pipe with a diameter of 21.3 mm and a wall thickness of 2.8 mm. The complex uses stainless steel fasteners. Protruding parts of threaded connections and open parts of pipes are closed with plastic plugs. Element coverage: - metal - powder zinc primer, powder paint; - timber - glazing primer, water-dispersion varnish. |  |  |  |