| 32.034 |  | Parking bollard |  |
| --- | --- | --- | --- |
| Size, mm: | 150x150x750 | Weight, kg: | 4.3793 |
| Material characteristics |  |  |  |
| Primary materials: stainless steel, stainless steel fasteners. Scope of supply: parking bollard – 1 pc.; fastener kit – 1 set. The bollard is manufactured from a stainless steel tube with an outer diameter of 76.1 mm and a wall thickness of 2 mm. The base flange is made from 4 mm thick stainless steel plate. The top cap is made from 2 mm thick stainless steel plate. The assembly utilizes stainless steel fasteners throughout. For safety and aesthetics, the exposed threads of all bolted connections are covered with plastic protective caps. |  |  |  |