| 16.052 |  | Spring Rider Stump |  |
| --- | --- | --- | --- |
| Size, mm: | 587x765x854 | Weight, kg: | 47.877 |
| Material characteristics |  |  |  |
| Materials: HPL plastic, metal, moisture-resistant laminated plywood. General description of the design The product is a rocking chair on a metal spring, where the walls are made of HPL plastic decorated with the shape of a tree leaf and slots in the form of leaves. Materials and components Powder-painted steel with a layer of zinc-containing primer, HPL panels, laminated waterproof plywood, stainless steel fasteners. The frame consists of the metal base of the rocking chair (platform-springs-platform), as well as straight and bent round metal pipes that determine the basic shape of the rocking chair. The seat is made of high-strength, moisture-resistant laminated plywood 15 mm thick, lined with phenolic or melamine film 120-220 g/m2, with a signature “hexagonal structure” pattern. The side walls are made of HPL plastic with a thickness of at least 10 mm. All edges are rounded using special knives or milling. Installed on two end compression springs. The spring is fastened to the support platform using U-shaped clamps. |  |  |  |