| LGTU-503 |  | Sports Station 503 |  |
| --- | --- | --- | --- |
| Size, mm: | 4772x5736x3491 | Weight, kg: | 402 |
| Material characteristics |  |  |  |
| Length 5736 * width 4772 * height 3491 mm. Weight 602 kg Purpose: The sports complex is designed for the development of all muscle groups and cardio training. Materials: Structural steel (Pipe 100*100*4 GOST8639, 80*80*3 GOST8639, DN25*2.8 GOST3262, DN32*2.8 GOST3262; sheet 2 GOST 19903, 4 GOST 19903, 6 GOST 19903, 8 GOST 19903; circle 10 GOST2590, 16 GOST2590, 30 GOST2590; corner 35x35x3 GOST8509). TRS plywood, EPDM coating (polyurethane), plastic elements (plugs, caps) - (PVC plastic 0-40 GOST 5960, polyethylene ND GOST 16338) Installation: Assembling the structure at the site of operation according to the instructions (included in the kit), concreting the supporting elements into the ground to a depth of 0.6 - 0.8 m. Coating of elements: Polymer coating of metal. In ZE version - zinc coating of metal elements; ZEC - zinc coating of metal elements and polymer coating. Lumber coating - Remmers system. Fasteners: Galvanized hardware. Elements of the complex: Metal stand – 4 pcs. Canopy horizontal bar – 1 set Canopy beams – 1 set Canopy handrail – 1 set Canopy press – 1 set Handle canopy – 2 sets Canopy horizontal bar-ladder – 1 set Extended stand – 2 pcs. Wall canopy – 1 set Platform canopy for pacing – 1 set Gymnastic ring canopy – 2 sets Rope – 1 set Set of hardware – 1 pc. |  |  |  |