| 40.028 |  | Meteor |  |
| --- | --- | --- | --- |
| Size, mm: | 192x192x3600 | Weight, kg: | 30.288 |
| Material characteristics |  |  |  |
| A post for a section with a gate is a collapsible structure (main and upper element) used to connect sections and align them along the same axis, as well as for installation of a gate. Each structural element is made of a profile metal pipe with a square section of 80 mm x 80 mm and a wall thickness of 3 mm, to which a connecting flange is welded, a 3mm thick sheet metal embed and two loops for hanging the gate are welded to the main element. The elements are fixed to each other using a bolted connection. The bolted connections are closed with plastic spherical caps on the nut side. All open parts of the pipes are closed with plastic plugs of the appropriate size. The pillars are painted with basalt-gray shagreen polymer paint. |  |  |  |