| 13.200 |  | Play System Neon |  |
| --- | --- | --- | --- |
| Size, mm: | 3254x4154x2361 | Weight, kg: | 305.97 |
| Material characteristics |  |  |  |
| Main materials: softwood timber, planed softwood board, laminated plywood with anti-slip coating, HPL plastic, stainless steel, reinforced polypropylene rope, stainless fasteners. Contents: embedded parts - 1 set, support posts - 7 pcs., floors - 1 set, straight slide h=1.5m - 1 pc., fencing and decorative panels - 1 set, - climber panels with grooves for legs - 1 pc., rope mesh - 1 pc., rope descents - 4 pcs., vertical pole - 1 pc., handles - 4 pcs., ties - 3 pcs., Tic-Tac-Toe panel - 1 pc., fastening kit - 1 pc. Design and color palette of equipment according to the sketch. The play complex consists of 1 platform and a rope net. The support posts are made of laminated softwood timber with a cross-section of 100x100 mm. The profile is square, the corner radius is R10. The rope mesh is made of six-strand reinforced polypropylene rope d=16 mm with a metal frame made of a metal profile pipe with a cross-section of 60x40 mm and a wall thickness of 2 mm. The floor frames are planed softwood boards with a section of 100x30 mm, 100x40 mm. The floors and steps are made of laminated plywood 18 mm thick with an anti-slip coating. Fencing and decorative panels, game elements are made of double-sided HPL plastic with UV protection, 10 mm thick. Drawings are applied to panels and game elements using engraving and UV printing. - climber panels with grooves for legs, made of HPL plastic 10 mm thick; The handles are made of metal pipe with a diameter of 26.8 mm and a wall thickness of 2.8 mm. Metal crossbars are made of a round pipe with a diameter of 21.3 mm and a wall thickness of 2.8 mm. The vertical pole is made of a pipe with a diameter of 42 mm and a wall thickness of 2.8 mm. Tic-tac-toe is made of laminated coniferous timber with a cross-section of 100x100 mm. The profile is square, the corner radius is R10. The metal plates are made of stainless steel with a thickness of 1.5 mm. The play complex includes 1 straight slide h=1.5 m. The slide consists of a slope and sides, in turn made of stainless steel sheet 1.5 mm thick and HPL plastic 10 mm thick. The launch pad for the straight slide is equipped with a metal crossbar made of a round pipe with a diameter of 21.3 mm and a wall thickness of 2.8 mm. The complex uses stainless steel fasteners. Protruding parts of threaded connections and open parts of pipes are closed with plastic plugs. Element coverage: - metal - powder zinc primer, powder paint; - timber - glazing primer, water-dispersion varnish. |  |  |  |